# Switch Abstraction Interface

# Change Proposal

| Title | Uniform tunnel API’s |
| --- | --- |
| Authors | Mellanox , metaSwitch |
| Status | First draft |
| Type | Standards Track |
| Created | 08/01/2015 |
| SAI-Version | V0.9.4 |

Contents

List of Changes	i

1	Overview	1

1.1	Uniform tunnel object	1

1.2	Uniform tunnel encap	1

1.3	IPinIP tunnel	3

1.3.1	IPinIP  encap	3

1.3.2	IPinIP  decap	3

1.4	Vxlan tunnel	4

1.4.1	Vxlan  encap	4

1.4.2	Vxlan  decap	4

1.5	MPLS tunnel	4

1.5.1	MPLS encap	5

1.5.2	MPLS  decap	5

2	Specification	6

2.1	SAI types	6

2.2	FDB	7

2.3	Next hop	7

2.4	Tunnel mapper	8

2.5	Tunnel object	10

2.6	Tunnel table entry object	13

3	Examples	16

3.1	IPinIP - point to point tunnel	16

3.2	VXlan	18

# List of Changes

| Version | Changes | Name | Date |
| --- | --- | --- | --- |
| 0.9.4 | Proposal for uniform tunnel |  | 8/1/15 |

License

© 2014 Microsoft Corporation, Dell Inc., Facebook, Inc, Broadcom Corporation, Intel Corporation, Mellanox Technologies Ltd.

As of September 9, 2014, the following persons or entities have made this Specification available under the Open Web Foundation Final Specification Agreement (OWFa 1.0), which is available at http://www.openwebfoundation.org/legal/the-owf-1-0-agreements/owfa-1-0

Microsoft Corporation, Dell Inc., Facebook, Inc, Intel Corporation, Mellanox Technologies Ltd.

You can review the signed copies of the Open Web Foundation Agreement Version 1.0 for this Specification at http://opencompute.org/licensing/, which may also include additional parties to those listed above.

Your use of this Specification may be subject to other third party rights. THIS SPECIFICATION IS PROVIDED "AS IS." The contributors expressly disclaim any warranties (express, implied, or otherwise), including implied warranties of merchantability, noninfringement, fitness for a particular purpose, or title, related to the Specification. The entire risk as to implementing or otherwise using the Specification is assumed by the Specification implementer and user. IN NO EVENT WILL ANY PARTY BE LIABLE TO ANY OTHER PARTY FOR LOST PROFITS OR ANY FORM OF INDIRECT, SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES OF ANY CHARACTER FROM ANY CAUSES OF ACTION OF ANY KIND WITH RESPECT TO THIS SPECIFICATION OR ITS GOVERNING AGREEMENT, WHETHER BASED ON BREACH OF CONTRACT, TORT (INCLUDING NEGLIGENCE), OR OTHERWISE, AND WHETHER OR NOT THE OTHER PARTY HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

THE FOLLOWING IS A LIST OF MERELY REFERENCED TECHNOLOGY: Microprocessor technology, semiconductor manufacturing technology, operating system technology (including without limitation networking operating system technology), emulation technology, graphics technology, video technology, integrated circuit packaging technology and the like, compiler technologies, object oriented technology, optical/RF communications technology including chip I/O and driver technology, bus technology, memory chip technology (including, without limitation, NAND memory, NOR memory, resistive RAM (RRAM), seek scan probe (SSP) memory, nonvolatile memory (including without limitation, memory based on chalcogenide materials, phase change memory (PCM), one or more stacked layers of memory cells, embedded PCM memories, non-volatile cache memory, solid state drives, SRAM, embedded DRAM, ferro-electric memory, and polymer memory)) and/or health-related and medical technology. IMPLEMENTATION OF THESE TECHNOLOGIES MAY BE SUBJECT TO THEIR OWN LEGAL TERMS.

# Overview

# Uniform tunnel object

# Uniform tunnel encap

Uniform tunnel decap option

The l3 tunnel table, is used in Oder to terminate tunnels that uses L3 as the underlay layer

L3 tunnel tables supports the following entry types

Point to point entry - Match on {VRF, DIP, SIP, tunnel type}    action -> tunnel id

Point to multi point entry - Match on {VRF, DIP, tunnel type}    action -> tunnel id

Default (no match) – forward to router

In case of overlap between Point to point entry & a Point to multi point entry the more specific entry (Point to point) will get precedence

## IPinIP tunnel

## IPinIP  encap

### IPinIP  decap

## Vxlan tunnel

### Vxlan  encap

### Vxlan  decap

## MPLS tunnel

### MPLS encap

### MPLS  decap

# Specification

New object needed:

Next hop:

Tunnel decap,

Tunnel encap,

MPLS next-hop(NHLFE)

Generic Tunnel object

typedef enum _sai_router_interface_type_t

{

/** Port or Lag Router Interface Type */

SAI_ROUTER_INTERFACE_TYPE_PORT,

/** VLAN Router Interface Type */

SAI_ROUTER_INTERFACE_TYPE_VLAN

/** VLAN Router Interface Type */

SAI_ROUTER_INTERFACE_TYPE_LOOPBACK

} sai_router_interface_type_t

## SAI types

typedef struct _sai_tunnel_map_params_t

{

/** ECN  */

sai_uint8_t ecn;

/** vlan id  */

sai_uint16_t vlan_id;

/** VNI id  */

sai_uint32_t vni_id;

} sai_tunnel_map_params_t;

typedef struct _sai_tunnel_map_t

{

/** Input parameters to match */

sai_tunnel_map_params_t key;

/** Output map parameters */

sai_tunnel_map_params_t value;

} sai_tunnel_map_t;

typedef struct _sai_tunnel_map_list_t

{

/** Number of entries in the map  */

uint32_t count;

/** Map list */

sai_tunnel_map_t * list;

} sai_tunnel_map_list_t;

## FDB

typedef enum _sai_fdb_entry_attr_t

{

/** FDB entry port id [sai_object_id_t] (MANDATORY_ON_CREATE|CREATE_AND_SET)

* The port id here can refer to a generic port object such as SAI port object id,

* SAI LAG object id and etc. Or to a tunnel next hop object in case the entry is l2 tunnel  */

SAI_FDB_ENTRY_ATTR_PORT_ID,

} sai_fdb_entry_attr_t;

## Next hop

/**

*  @brief Next hop type

*/

typedef enum _sai_next_hop_type_t

{

SAI_NEXT_HOP_IP,

/*MPLS(NHLFE) next hop */

SAI_NEXT_HOP_MPLS,

/*tunnel next hop */

SAI_NEXT_HOP_TUNNEL_DECAP,

SAI_NEXT_HOP_TUNNEL_ENCAP,

} sai_next_hop_type_t;

typedef enum _sai_next_hop_attr_t

{

/** READ-ONLY */

/** READ-WRITE */

/** Next hop entry type [sai_next_hop_type_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_TYPE,

/** Next hop entry ipv4 address [sai_ip_address_t]

* (MANDATORY_ON_CREATE when SAI_NEXT_HOP_ATTR_TYPE = SAI_NEXT_HOP_IP)

* (CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_IP,

/** Next hop entry router interface id [sai_object_id_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_ROUTER_INTERFACE_ID,

/* -- */

/** Next hop entry tunnel-dst [sai_object_id_t]

* (MANDATORY_ON_CREATE when SAI_NEXT_HOP_ATTR_TYPE=SAI_NEXT_HOP_TUNNEL_ENCAP)

* (CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_TUNNEL_DST,

/** Next hop entry tunnel-id [sai_object_id_t]

* (MANDATORY_ON_CREATE when SAI_NEXT_HOP_ATTR_TYPE = SAI_NEXT_HOP_TUNNEL_DECAP | SAI_NEXT_HOP_TUNNEL_ENCAP)

* (CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_TUNNEL_ID,

/** Custom range base value */

SAI_NEXT_HOP_ATTR_CUSTOM_RANGE_BASE  = 0x10000000

} sai_next_hop_attr_t;

## } sai_next_hop_api_t;

## Tunnel mapper

*/

/**

* @brief Enum defining tunnel map types.

*/

typedef enum _sai_tunnel_map_type_t

{

/** TUNNEL Map overlay ECN to underlay ECN   */

SAI_TUNNEL_MAP_OECN_TO_UECN = 0x00000001,

/** TUNNEL Map underlay ECN to overlay ECN   */

SAI_TUNNEL_MAP_UECN_TO_OECN = 0x00000002,

/** TUNNEL Map VNI to VLAN ID  */

SAI_TUNNEL_MAP_VNI_TO_VLAN_ID = 0x00000003,

/** TUNNEL Map VLAN ID to VNI */

SAI_TUNNEL_MAP_VNI_TO_VLAN_ID = 0x00000004,

/* -- */

/* Custom range base value */

SAI_TUNNEL_MAP_CUSTOM_RANGE_BASE = 0x10000000

} sai_tunnel_map_type_t

typedef enum _sai_tunnel_map_attr_t

{

/** tunnel Map type [sai_tunnel_map_type_t](MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_TUNNEL_MAP_ATTR_TYPE = 0x00000000,

/** tunnel mapper  [sai_tunnel_map_list_t],

*/

SAI_TUNNEL_MAP_ATTR_MAP_TO_VALUE_LIST = 0x00000001,

/* -- */

/* Custom range base value */

SAI_TUNNEL_MAP_ATTR_CUSTOM_RANGE_BASE = 0x10000000

} sai_tunnel_map_attr_t ;

/**

* @brief Create tunnel Map

*

* @param[out] tunnel_map_id tunnel Map Id

* @param[in] attr_count number of attributes

* @param[in] attr_list array of attributes

*

* @return  SAI_STATUS_SUCCESS on success

*          Failure status code on error

*/

typedef sai_status_t (*sai_create_tunnel_map_fn)(

_Out_ sai_object_id_t* tunnel_map_id,

_In_ uint32_t attr_count,

_In_ const sai_attribute_t *attr_list

);

/**

* @brief Remove tunnel Map

*

*  @param[in] tunnel_map_id tunnel Map id to be removed.

*

*  @return  SAI_STATUS_SUCCESS on success

*           Failure status code on error

*/

typedef sai_status_t (*sai_remove_tunnel_map_fn) (

_In_  sai_object_id_t   tunnel_map_id

);

/**

* @brief Set attributes for tunnel map

*

* @param[in] tunnel_map_id tunnel Map Id

* @param[in] attr attribute to set

*

* @return  SAI_STATUS_SUCCESS on success

*          Failure status code on error

*/

typedef sai_status_t (*sai_set_tunnel_map_attribute_fn)(

_In_ sai_object_id_t  tunnel_map_id,

_In_ const sai_attribute_t *attr

);

/**

* @brief  Get attrbutes of tunnel map

*

* @param[in] tunnel_map_id  tunnel map id

* @param[in] attr_count  number of attributes

* @param[inout] attr_list  array of attributes

*

* @return SAI_STATUS_SUCCESS on success

*        Failure status code on error

*/

typedef sai_status_t (*sai_get_tunnel_map_attribute_fn)(

_In_ sai_object_id_t   tunnel_map_id ,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

/**

*  tunnel Map methods table retrieved with sai_api_query()

*/

typedef struct _sai_tunnel_map_api_t

{

sai_create_tunnel_map_fn         create_tunnel_map;

sai_tunnel_tunnel_map_fn         remove_tunnel_map;

sai_set_tunnel_map_attribute_fn  set_tunnel_map_attribute;

sai_get_tunnel_map_attribute_fn  get_tunnel_map_attribute;

}sai_tunnel_map_api_t;

## Tunnel object

typedef enum _sai_tunnel_type_t

{

SAI_TUNNEL_IPINIP,

SAI_TUNNEL_IPINIP_GRE,

SAI_TUNNEL_IPINIP_GRE_AND_KEY

SAI_TUNNEL_VXLAN,

SAI_TUNNEL_MPLS,

…

} sai_tunnel_type_t;

typedef enum _sai_tunnel_ttl_mode_t

{

SAI_TUNNEL_TTL_COPY_FROM_INNER,

SAI_TUNNEL_TTL_USER_DEFINE

} sai_tunnel_ttl_mode_t

typedef enum _sai_tunnel_dscp_mode_t

{

SAI_TUNNEL_DSCP_COPY_FROM_INNER,

SAI_TUNNEL_DSCP_USER_DEFINE

} sai_tunnel_dscp_mode_t

typedef enum _sai_tunnel_id_t

{

/** READ-WRITE */

/** tunnel type [sai_tunnel_type_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_TUNNEL_ATTR_TYPE;

/** tunnel underlay interface [sai_object_id_t] */

SAI_TUNNEL_UNDERLAY_INTERFACE;

/** tunnel overlay interafce [sai_object_id_t] */

SAI_TUNNEL_OVERLAY_INTERFACE;

/** tunnel encap attribute*/

/** tunnel ip verssion ipv4/ipv6  (MANDATORY_ON_CREATE when (CREATE_ONLY) */

SAI_TUNNEL_IP_VER,

/**  tunnel src ip (MANDATORY_ON_CREATE when     SAI_TUNNEL_ATTR_TYPE=SAI_TUNNEL_IPINIP,SAI_TUNNEL_IPINIP_GRE,SAI_TUNNEL_IPINIP_GRE_AND_KEY)

(CREATE_ONLY) */

SAI_TUNNEL_SRC_IP,

/**  tunnel TTL mode (copy from inner or user define  [sai_tunnel_ttl_mode_t] MANDATORY_ON_CREATE when  SAI_TUNNEL_ATTR_TYPE=SAI_TUNNEL_IPINIP,SAI_TUNNEL_IPINIP_GRE,SAI_TUNNEL_IPINIP_GRE_AND_KEY)

(CREATE_ONLY) */

SAI_TUNNEL_ENCAP_TTL_MODE,

/**  tunnel TTL value  MANDATORY_ON_CREATE when  SAI_TUNNEL_TTL_MODE = SAI_TUNNEL_TTL_USER_DEFINE)

SAI_TUNNEL_ENCAP_TTL_VAL,

/**  tunnel dscp  mode (pipe or uniform model )  [sai_tunnel_dscp_mode_t] MANDATORY_ON_CREATE when  SAI_TUNNEL_ATTR_TYPE=SAI_TUNNEL_IPINIP,SAI_TUNNEL_IPINIP_GRE,SAI_TUNNEL_IPINIP_GRE_AND_KEY)

(CREATE_ONLY) */

SAI_TUNNEL_ENCAP_DSCP_MODE,

/**  tunnel DSCP value  MANDATORY_ON_CREATE when  SAI_TUNNEL_DSCP_MODE = SAI_TUNNEL_DSCP_USER_DEFINE)

SAI_TUNNEL_ENCAP_DSCP_VAL,

/** tunnel GEP key (MANDATORY_ON_CREATE when SAI_TUNNEL_ATTR_TYPE=SAI_TUNNEL_IPINIP_GRE_AND_KEY)

(CREATE_ONLY) */

*/

SAI_TUNNEL_ENCAP_GRE_KEY,

/**  tunnel mapper [sai_object_id_t] */

SAI_TUNNEL_ENCAP_MAPPER,

/** tunnel dencap attribute **/

/**  enable decap src ip validation check  MANDATORY_ON_CREATE when     SAI_TUNNEL_ATTR_TYPE=SAI_TUNNEL_IPINIP,SAI_TUNNEL_IPINIP_GRE,SAI_TUNNEL_IPINIP_GRE_AND_KEY)

*/

SAI_TUNNEL_DECAP_SIP_CHECK,

/**  expected tunnel src ip (MANDATORY_ON_CREATE when  SAI_TUNNEL_DECAP_SIP_CHECK is enabled)

(CREATE_ONLY) */

SAI_TUNNEL_DECAP_EXPECTED_SRC_IP,

/**  tunnel decap ECN mapping [sai_object_id_t] */

SAI_TUNNEL_DECAP_ECN_TABLE,

/**  tunnel mapper [sai_object_id_t] */

SAI_TUNNEL_DECAP_MAPPER,

} sai_tunnel_id_t;

/**

* Routine Description:

*    @brief Create tunnel

*

* Arguments:

*    @param[out] tunnel_id - tunnel id

*    @param[in] attr_count - number of attributes

*    @param[in] attr_list - array of attributes

*

* Return Values:

*    @return SAI_STATUS_SUCCESS on success

*            Failure status code on error

*

*/

typedef sai_status_t (*sai_create_tunnel_fn)(

_Out_ sai_object_id_t* tunnel_id,

_In_ uint32_t attr_count,

_In_ const sai_attribute_t *attr_list

);

/**

* Routine Description:

*    @brief remove  tunnel

*

* Arguments:

*    @param[in] tunnel_id – tunnel id

*

* Return Values:

*    @return SAI_STATUS_SUCCESS on success

*            Failure status code on error

*/

typedef sai_status_t (*sai_remove_tunnel_fn)(

_In_ sai_object_id_t tunnel_id

);

/**

* Routine Description:

*    @brief Set tunnel attribute

*

* Arguments:

*    @param[in] tunnel_id - tunnel id

*    @param[in] attr - attribute

*

* Return Values:

*    @return SAI_STATUS_SUCCESS on success

*            Failure status code on error

*/

typedef sai_status_t (*sai_set_tunnel_attribute_fn)(

_In_ sai_object_id_t tunnel_id,

_In_ const sai_attribute_t *attr

);

/**

* Routine Description:

*    @brief Get tunnel attribute

*

* Arguments:

*    @param[in] tunnel _id - tunnel id

*    @param[in] attr_count - number of attributes

*    @param[inout] attr_list - array of attributes

*

* Return Values:

*    @return SAI_STATUS_SUCCESS on success

*            Failure status code on error

*/

typedef sai_status_t (*sai_get_tunnel_attribute_fn)(

_In_ sai_object_id_t tunnel_id,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

/**

*  @brief tunnel  methods table retrieved with sai_api_query()

*/

typedef struct _sai_tunnel_api_t

{

sai_create_tunnel_fn        create_tunnel;

sai_remove_tunnel_fn        remove_tunnel;

sai_set_tunnel_attribute_fn set_tunnel_attribute;

sai_get_tunnel_attribute_fn get_tunnel_attribute;

}

## Tunnel table entry object

typedef enum _sai_tunnel_table_entry_type_t

{

/** tunnel table point to point entry match on dst & src ip & tunnel type  **/

SAI_TUNNEL_TABLE_ENTRY_P2P,

/** tunnel table point to multi point entry match on dst ip  & tunnel type  **/

SAI_TUNNEL_TABLE_ENTRY_P2MP,

} sai_tunnel_table_entry_type_t;

typedef enum _sai_tunnel_table_entry_t;

{

/** READ-ONLY */

/** READ-WRITE */

/** tunnel virtual router id [sai_object_id_t] (MANDATORY_ON_CREATE|CREATE_ONLY)*/

SAI_TUNNEL_ENTRY_FIELD_VR_ID,

/** tunnel entry type    [sai_tunnel_table_entry_type_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_TUNNEL_ENTRY_TYPE,

/** tunnel termination ip address   [sai_ip_address_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_TUNNEL_ENTRY_FIELD_DST_IP,

/** tunnel source ip address   [sai_ip_address_t] (MANDATORY_ON_CREATE|CREATE_ONLY) valid onlt for  SAI_TUNNEL_TABLE_P2P  */

SAI_TUNNEL_ENTRY_FIELD_SRC_IP,

/** tunnel  type    [sai_tunnel_type_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_TUNNEL_ENTRY_FIELD_TUNNEL_TYPE;

/** tunnel id to be use for decap [sai_tunnel_id_t] (MANDATORY_ON_CREATE|CREATE_ONLY) valid o

SAI_TUNNEL_ENTRY_ACTION_TUNNEL_ID,

/** Custom range base value */

SAI_TUNNEL_TABLE_ENTRY_CUSTOM_RANGE_BASE  = 0x10000000

} sai_tunnel_table_entry_t;;

/**

* Routine Description:

*    @brief Create tunnel table entry

*

* Arguments:

*    @param[out] tunnel_table_entry_id - tunnel_table_entry_id

*    @param[in] attr_count - number of attributes

*    @param[in] attr_list - array of attributes

*

* Return Values:

*    @return SAI_STATUS_SUCCESS on success

*            Failure status code on error

*

* Note: IP address expected in Network Byte Order.

*/

typedef sai_status_t (*sai_create_tunnel_table_entry_fn)(

_Out_ sai_object_id_t* tunnel_table_entry_id,

_In_ uint32_t attr_count,

_In_ const sai_attribute_t *attr_list

);

/**

* Routine Description:

*    @brief Remove next hop

*

* Arguments:

*    @param[in] remove tunnel table entry

*

* Return Values:

*    @return SAI_STATUS_SUCCESS on success

*            Failure status code on error

*/

typedef sai_status_t (*sai_remove_tunnel_fn)(

_In_ sai_object_id_t tunnel_id

);

/**

* Routine Description:

*    @brief tunnel table entry attribute

*

* Arguments:

*    @param[in] tunnel_id - tunnel id

*    @param[in] attr - attribute

*

* Return Values:

*    @return SAI_STATUS_SUCCESS on success

*            Failure status code on error

*/

typedef sai_status_t (*sai_set_tunnel_attribute_fn)(

_In_ sai_object_id_t tunnel_table_entry_id,

_In_ const sai_attribute_t *attr

);

/**

* Routine Description:

*    @brief Get tunnel table entry attribute

*

* Arguments:

*    @param[in] tunnel_table_entry_id – tunnel table entry id

*    @param[in] attr_count - number of attributes

*    @param[inout] attr_list - array of attributes

*

* Return Values:

*    @return SAI_STATUS_SUCCESS on success

*            Failure status code on error

*/

typedef sai_status_t (*sai_get_tunnel_attribute_fn)(

_In_ sai_object_id_t tunnel_table_entry_id,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

/**

*  @brief tunnel table entry  retrieved with sai_api_query()

*/

typedef struct _sai_tunnel_table_entry_api_t

{

sai_create_tunnel_table_entry_fn        create_tunnel_table_entry;

sai_remove_tunnel_table_entry_fn        remove_tunnel_table_entry;

sai_set_tunnel_table_entry_attribute_fn set_tunnel_table_entry_attribute;

sai_get_tunnel_table_entry_attribute_fn get_tunnel_table_entry_attribute;

} sai_tunnel_table_entry_api_t;

## Examples

## IPinIP - point to point tunnel

## Setup overview

Two ip networks connected via point to point ipinip tunnel details in the diagram below

## Zoom in to the left side for the tunnel

## Two ip networks connected via point to point ipinip tunnel details in the diagram below

device A configuration

Two virtual_router ,  underlay VRF, overlay VRF

overlay VRF router interface configuration

router interface rif_a type vlan , vlan 1

router interface rif_lb1 type loopback (new type SAI_ROUTER_INTERFACE_TYPE_LOOPBACK)

local network  route 192.168.0.1->rif_a

underlay VRF router interface configuration

router interface rif_lb2 type loopback (new type SAI_ROUTER_INTERFACE_TYPE_LOOPBACK)

local ip route 2.2.2.3->rif_lb2

tunnel configuration :

tunnel   ipinip_obj

SAI_TUNNEL_ATTR_TYPE = SAI_TUNNEL_IPINIP

SAI_TUNNEL_OVERLAY_INTERFACE= rif_lb1

SAI_TUNNEL_UNDERLAY_INTERFACE= rif_lb2

SAI_TUNNEL_ENCAP_ATTR=

{

SAI_TUNNEL_IP_VER=ipv4

SAI_TUNNEL_SRC_IP=2.2.2.3

SAI_TUNNEL_TTL_MODE= SAI_TUNNEL_TTL_COPY_FROM_INNER

SAI_TUNNEL_DSCP_MODE= SAI_TUNNEL_DSCP_COPY_FROM_INNER

}

SAI_TUNNEL_DECAP_ATTR

{

SAI_TUNNEL_DECAP_SIP_CHECK=NON

}

Tunnel table configuration:

tunnel  table entry  ipinip_decap

SAI_TUNNEL_ENTRY_TYPE = SAI_TUNNEL_TABLE_ENTRY_P2P;

SAI_TUNNEL_ENTRY_FEILD_VR_ID= underlay VRF

SAI_TUNNEL_ENTRY_FIELD_DST_IP=2.2.2.3 ;

SAI_TUNNEL_ENTRY_FIELD_SRC_IP=3.3.3.3 ;

SAI_TUNNEL_ENTRY_FIELD_TUNNEL_TYPE= SAI_TUNNEL_IPINIP

SAI_TUNNEL_ENTRY_ACTION_TUNNEL_ID= ipinip_obj

Next hop configuration :

Next hop    ipinip_encap_obj

{

SAI_NEXT_HOP_ATTR_TYPE= SAI_NEXT_HOP_TUNNEL_ENCAP(new type)

SAI_NEXT_HOP_ATTR_TUNNEL_ID= ipinip_obj

SAI_NEXT_HOP_ATTR_IP=3.3.3.3

}

Route configuration:

overlay VRF , 192.167.0.0/16 ->{ SAI_ROUTE_ATTR_PACKET_ACTION= FORWARD ,

SAI_ROUTE_ATTR_NEXT_HOP_ID= ipinip_encap_obj

}

## VXlan

## Setup overview

Overlay (virtual) network: Three tenants VA, VB, VC each tenants confine two l2 domain ( Vlans )

VA = VNI 2(map to vlan 2),VNI 3 (map to vlan 3)

Vlan 2 ip subnet = 192.168.0.0/16

Vlan 3 ip subnet = 192.167.0.0/16

VB = VNI 4(map to vlan 4),VNI 5 (map to vlan 5)

Vlan 4 ip subnet = 192.168.0.0/16

Vlan 5 ip subnet = 192.167.0.0/16

VC = VNI 400(map to vlan 6),VNI 500 (map to vlan 7)

Vlan 6 ip subnet = 192.168.0.0/16

Vlan 2 ip subnet = 192.166.0.0/16

(Since SAI doesn’t support defining of bridging domain we assume bridging domain -== vlan id -> tenants can’t share Vlan

Underlay (physical) network:

The virtual network is spared across three phy location connected via l3 network

Two are PC base hypervisor the gust Overlay (virtual) network is running on the VM on those hosts

The third location is bare metal hosts connect to a switch with VTEP endpoint

The network is described in the diagram below;

## Zoom in to device A network

device A configuration

4 virtual_router

underlay VRF,

overlay VRF for tenant VA_VRF

overlay VRF for tenant VB_VRF

overlay VRF for tenant VC_VRF

overlay VRF_VA router interface configuration

router interface rif_va_1 type vlan , vlan 2

router interface rif_va_2 type vlan , vlan 3

local network  route 192.168.0.0/16-> rif_va_1

local network  route 192.167.0.0/16-> rif_va_2

overlay VRF_VB router interface configuration

router interface rif_vb_1 type vlan , vlan 4

router interface rif_vb_2 type vlan , vlan 5

local network  route 192.168.0.0/16-> rif_vb_1

local network  route 192.167.0.0/16-> rif_vb_2

overlay VRF_VC router interface configuration

router interface rif_va_1 type vlan , vlan 5

router interface rif_va_2 type vlan , vlan 6

local network  route 192.168.0.0/16-> rif_vc_1

local network  route 192.166.0.0/16-> rif_vc_2

underlay VRF router interface configuration

router interface rif_lb2 type loopback (new type SAI_ROUTER_INTERFACE_TYPE_LOOPBACK)

local ip route 4.4.4.3/32->rif_lb1

tunnel mapper configuration // in order to be able to map VNI to vlan and vise versa

tunnel mapper   vlan_to_vni_map

SAI_TUNNEL_MAP_ATTR_TYPE= SAI_TUNNEL_MAP_VLAN_ID_TO_VNI

SAI_TUNNEL_MAP_ATTR_MAP_TO_VALUE_LIST

{

Count=6

{key=2,value=2},{key=3,value=3},{key=4,value=4},

{key=5,value=5},{key=6,value=400},{key=7,value=500}

}

tunnel mapper   vni_to_vlan_map

SAI_TUNNEL_MAP_ATTR_TYPE= SAI_TUNNEL_MAP_VNI_TO_VLAN_ID

SAI_TUNNEL_MAP_ATTR_MAP_TO_VALUE_LIST

{

Count=6

{key=2,value=2},{key=3,value=3},{key=4,value=4},

{key=5,value=5},{key=400,value=6},{key=500,value=7}

}

tunnel configuration :

tunnel   vxlan_obj

SAI_TUNNEL_ATTR_TYPE = SAI_TUNNEL_VXLAN

SAI_TUNNEL_OVERLAY_INTERFACE= NULL(should be the bridge port connected to the tunnel currently missing form SAI)

SAI_TUNNEL_UNDERLAY_INTERFACE= rif_lb1

SAI_TUNNEL_ENCAP_ATTR=

{

SAI_TUNNEL_IP_VER=ipv4

SAI_TUNNEL_IP_VER

SAI_TUNNEL_SRC_IP=4.4.4.3

SAI_TUNNEL_TTL_MODE= SAI_TUNNEL_TTL_USER_DEFINE

SAI_TUNNEL_TTL_VAL=1

SAI_TUNNEL_DSCP_MODE= SAI_TUNNEL_DSCP_USER_DEFINE

SAI_TUNNEL_DSCP_VAL=7

SAI_TUNNEL_ENCAP_MAPPER= vlan_to_vni_map

}

SAI_TUNNEL_DECAP_ATTR

{

SAI_TUNNEL_DECAP_MAPPER= vni_to_vlan_map

}

Tunnel table configuration:

tunnel  table entry  vxlan_decap

SAI_TUNNEL_ENTRY_TYPE = SAI_TUNNEL_TABLE_ENTRY_P2MP;

SAI_TUNNEL_ENTRY_FEILD_VR_ID= underlay VRF

SAI_TUNNEL_ENTRY_FIELD_DST_IP=4.4.4.3

SAI_TUNNEL_ENTRY_FIELD_TUNNEL_TYPE= SAI_TUNNEL_VXLAN

SAI_TUNNEL_ENTRY_ACTION_TUNNEL_ID= vxlan_obj

Next hop configuration :

Next hop    _encap_obj_host_a

{

SAI_NEXT_HOP_ATTR_TYPE= SAI_NEXT_HOP_TUNNEL_ENCAP(new type)

SAI_NEXT_HOP_ATTR_TUNNEL_ID= vxlan_obj

SAI_NEXT_HOP_ATTR_IP=2.2.2.3

}

Next hop    _encap_obj_host_b

{

SAI_NEXT_HOP_ATTR_TYPE= SAI_NEXT_HOP_TUNNEL_ENCAP(new type)

SAI_NEXT_HOP_ATTR_TUNNEL_ID= vxlan_obj

SAI_NEXT_HOP_ATTR_IP=3.3.3.3

}

FBD configuration:

MAC VM1,vlan 2 -> encap_obj_host_a

MAC VM2,vlan 4 -> encap_obj_host_a

MAC VM1,vlan 6 -> encap_obj_host_b

MAC VM2,vlan 5-> encap_obj_host_b